BEWARE

Lots of wet paper is heavy, do not overload trays.

Dyes can run and stain.

HANDlING

Wear gloves

Use Melinex to remove paper from water, slide underneath.

Place damp paper flat on a sheet or in a bread try, using sheet or tray for handling.

Wet paper is very fragile, keep handling to a minimum.

PROTECTION IN-SITU

(If items cannot be removed)

Place near walls and cover with polythene.

Order of Removal to Safe Area

Priority Items from room/s of greatest risk.

Remainder of items.

Paper - Salvage

WET

Air-dry flat, interleaved with blotting paper, either as individual sheets or in small piles (5 mm high). Change blotting paper as soon as wet.

Do NOT unfold or separate sheets that are stuck together or very wet.

OR consider freezing, see below.

Lightweight pamphlets can be hung from a line with plastic pegs.

Separate WET FROM DRY

Dry

Take to dry area and keep dry.

Put in sturdy, labelled crates.

Store in safe place.

FREEZE IF WET?

YES – if large volume of wet paper or hand coloured prints or inks are bleeding.

Separate with freezer paper and freeze immediately.

wet framed paper

Remove from frame in a safe, dry place unless stuck to glass. If paper is stuck leave on glass dry horizontally, glass side down.

If paper can be removed, dry as for wet paper.

Bag and label any tacks, pieces of frame or cord.

Paper - First Aid