BEWARE

Metals objects can be heavy.

Check electricity supply to lighting is disconnected.

HANDlING

Wear gloves.

Check priority sheets for details of chandelier or lantern hanging systems.

Pack small metal objects into crates, separating with bubble wrap.

As a last resort, wrought metal can be thrown out of windows.

Metals - Salvage

PROTECTION IN-SITU

(If items cannot be removed)

Fixed and large items should be covered with polythene.

If items can be moved, place on polythene by walls and cover with polythene

Order of Removal to Safe Area

Priority Items from room/s of greatest risk.

Remainder of items.

Door furniture.

WET

Remove excess water by blotting with kitchen towel.

Do NOT blot if there is an applied finish; air dry keeping flaking surfaces horizontal.

Air-dry as quickly as possible, unless there is an organic component, then air dry slowly.

Small items can be placed in a sealed box with silica gel.

Separate WET FROM DRY

FREEZE IF WET?

NO

Metals - First Aid

Dry

- Take to dry area and keep dry.

- Wrap in acid-free tissue and pack into labelled crates.

- Store in safe place, preferably with a low relative humidity.